| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/CHE/81 5 February 2007 |
|  | (07-0477) |
| Committee on Technical Barriers to Trade | Original: English |

NOTIFICATION

The following notification is being circulated in accordance with Article 10.6.

| 1. | Member to Agreement notifying: SWITZERLAND If applicable, name of local government involved (Articles 3.2 and 7.2): |  |  |
| --- | --- | --- | --- |
| 2. | Agency responsible: Federal Office for the Environment, CH-3003 Berne Name and address (including telephone and fax numbers, e-mail and web-site addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: State Secretariat for Economic Affairs Non-tariff Measures Division Effingerstrasse 1 CH-3003 Berne Tel: (+41) 31 324 07 60 Fax: (+41) 31 324 09 59 e-mail: faminia.depourtales@seco.admin.ch |  |  |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |  |  |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Coal and wood boilers; coal and wood stoves; industrial and commercial constructions which are sources of particulate emissions. |  |  |
| 5. | Title, number of pages and language(s) of the notified document: Draft revision of the Ordinance on Air Pollution Control (OAPC, Amendment of 1 July 2007.) (13 pages, in German, French and Italian) |  |  |
| 6. | Description of content: Measures to combat particulates: tighter limit values for wood- and coal-fired systems and further adaptations in this context: - Wood-fired systems with a thermal output up to 350 kW may only be introduced if they can be shown to be in conformity with the corresponding EU product standards and that they meet modern air pollution control requirements. - Automatic wood-fired systems with a thermal output in excess of 70kW must be fitted with effective particulate traps such as electrostatic dust precipitators or fibre filters. The new particulate limit values will be introduced progressively according to the size of the installation (2007 to 2015). - The general total particulate emission limit value of 50 mg/m3 will be reduced to 20 mg/m3 (analogous to the TA Luft in Germany). |  |  |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: In order to reduce air pollution due to particulates and to comply with the emission limit value specified by the Ordinance on Air Pollution Control, which today is exceeded on a regular basis. The particulate limit value for various equipment will be adapted to state of the art requirements. |  |  |
| 8. | Relevant documents: Ordinance on Air Pollution Control (OAPC) of 16 December 1985, SR 814.318.142.1 |  |  |
| 9. | Proposed date of adoption: Proposed date of entry into force: |  | May 2007 1 July 2007 (with a transitional period until 31 December 2007). |
| 10. | Final date for comments: 30 April 2007 |  |  |
| 11. | Texts available from: National enquiry point [ X ] or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: The text can be downloaded from the internet: German: http://www.news-service.admin.ch/NSBSubscriber/message/attachments/4136.pdf French: http://www.news-service.admin.ch/NSBSubscriber/message/attachments/4138.pdf Italian: http://www.news-service.admin.ch/NSBSubscriber/message/attachments/4140.pdf |  |  |